Suryanagar Refinery, Gujarat

HSE Incident / Near-Miss Report

Classification: Minor Environmental Incident (Contained)

# Minor Naphtha Spill During P-101 Seal-Related Work

| Report Number | INC-2026-0019 |
| --- | --- |
| Equipment Tag | P-101 |
| Equipment Description | Centrifugal Feed Pump, Unit 2 |
| Location | Unit 2 Pump Bay, bunded area around P-101 |
| Date of Incident | 2026-04-15 |
| Reported By | M. Iyer (Maintenance Head) |
| Investigated By | A. Nair (Safety Officer) |
| Linked Documents | Work Order WO-2026-0098 (P-101 Seal Leak Repair); Inspection Report INS-2026-0031 (P-101 Seal Wear Follow-up) |

## 1. Description of Event

While preparing P-101 for a follow-up seal inspection related to the wear findings in Inspection Report INS-2026-0031, a small quantity (estimated under 5 litres) of naphtha was released from the pump casing drain during depressurization. The release was fully contained within the bunded area around the pump.

## 2. Immediate Actions Taken

- Area cordoned off and spill contained within existing bund.

- Spill absorbed using standard spill kit materials.

- Pump isolated and confirmed depressurized before further work continued.

## 3. Root Cause / Contributing Factors

Casing drain valve was assumed closed based on standard procedure but was not visually confirmed before depressurization began. No environmental release outside the bund occurred.

## 4. Corrective / Preventive Actions

- Reinforce requirement to visually confirm drain valve position before depressurization, not rely on assumption.

- Add explicit drain valve check step to P-101 startup/shutdown SOP (SOP-UNIT2-014).

- No further action required on containment; bund performed as designed.

Safety Officer Sign-off: ____________________   Date: ____________________