Suryanagar Refinery, Gujarat

HSE Incident / Near-Miss Report

Classification: Near-Miss (No Injury)

# Near-Miss - Scaffold Instability During V-204 Repair

| Report Number | INC-2026-0032 |
| --- | --- |
| Equipment Tag | V-204 |
| Equipment Description | Naphtha Storage Tank, 5000 kL capacity |
| Location | Tank Farm, Unit 2 Battery Limit - V-204 shell base |
| Date of Incident | 2026-05-12 |
| Reported By | S. Verma (Field Technician) |
| Investigated By | A. Nair (Safety Officer) |
| Linked Documents | Hot Work Permit HWP-2026-0057; Work Order WO-2026-0142 (V-204 Corrosion Repair) |

## 1. Description of Event

During ongoing shell repair work on V-204 under Hot Work Permit HWP-2026-0057, a technician noticed lateral movement in the scaffold erected at the tank base while adjusting a weld position. Work was immediately stopped. No fall or contact occurred and no injury resulted.

## 2. Immediate Actions Taken

- Work stopped immediately and personnel moved away from scaffold structure.

- Scaffold inspected on the spot and found to have a loose base plate connection.

- Base plate connection re-secured and scaffold re-inspected before work resumed.

## 3. Root Cause / Contributing Factors

Scaffold base plate was not fully torqued during original erection, likely due to time pressure to begin hot work on schedule. Pre-use scaffold inspection checklist was completed but did not specifically verify base plate torque.

## 4. Corrective / Preventive Actions

- Update scaffold pre-use inspection checklist to include explicit base plate torque verification.

- Brief scaffold erection contractor on findings and reinforce inspection sign-off requirements.

- Flag for QA review whether this event and its link to Work Order WO-2026-0142 was documented within the required timeframe.

Safety Officer Sign-off: ____________________   Date: ____________________