Suryanagar Refinery, Gujarat

Equipment Inspection Report

# P-101 Mechanical Seal Wear Follow-up Inspection

| Report Number | INS-2026-0031 |
| --- | --- |
| Equipment Tag | P-101 |
| Equipment Description | Centrifugal Feed Pump, Unit 2 |
| Location | Unit 2 Pump Bay |
| Inspection Date | 2026-03-05 |
| Inspected By | D. Rao (QA / Compliance Officer) |
| Reviewed By | M. Iyer (Maintenance Head) |
| Linked Documents | Follow-up to Work Order D07 (WO-2026-0098); referenced by Incident Report D19 (2026-04-15) |

## 1. Scope of Inspection

Follow-up inspection of P-101 mechanical seal assembly following seal replacement carried out under Work Order D07, to verify root cause of the original leak.

## 2. Findings

| # | Observation | Severity | Recommendation |
| --- | --- | --- | --- |
| 1 | Wear pattern on replaced seal face consistent with shaft misalignment rather than flush line blockage | Medium | Perform shaft alignment check at next planned shutdown |
| 2 | Seal flush line found clear, no blockage confirmed | N/A | No action required on flush line |
| 3 | Baseplate bolt torque within specification | N/A | No action required |

## 3. Conclusion

The original seal leak on P-101 was most likely caused by shaft misalignment rather than flush line blockage as initially suspected in Work Order D07. Continued operation without correcting alignment may lead to recurrence.

## 4. Recommendation

Schedule a laser shaft alignment check on P-101 during the next planned shutdown. Monitor seal condition closely in the interim.

Inspector Signature: ____________________

Date: ____________________