Suryanagar Refinery, Gujarat

Equipment Inspection Report

# V-204 External Shell Corrosion Inspection

| Report Number | INS-2026-0011 |
| --- | --- |
| Equipment Tag | V-204 |
| Equipment Description | Naphtha Storage Tank, 5000 kL capacity |
| Location | Tank Farm, Unit 2 Battery Limit |
| Inspection Date | 2026-01-18 |
| Inspected By | S. Verma (Field Technician) |
| Reviewed By | D. Rao (QA / Compliance Officer) |
| Linked Documents | Referenced by Work Order D06 (WO-2026-0142); OISD-105 Audit D22 (2026-06-10) |

## 1. Scope of Inspection

Routine annual external visual inspection of tank V-204 shell, foundation, and nozzle connections, carried out as part of the scheduled storage tank integrity program.

## 2. Findings

| # | Observation | Severity | Recommendation |
| --- | --- | --- | --- |
| 1 | Moderate corrosion observed on lower shell course, external surface, approx. 1.2m section near nozzle N-4 | Medium | Grit blast, recoat, and assess remaining wall thickness |
| 2 | Coating breakdown on foundation ring, localized | Low | Schedule recoating during next maintenance window |
| 3 | No abnormal foundation settlement observed | N/A | No action required |

## 3. Conclusion

Tank V-204 remains fit for continued service. The corrosion identified on the lower shell course requires remedial action within 60 days to prevent further wall loss.

## 4. Recommendation

Raise a work order for corrosion repair (grit blast, thickness check, recoat) on the affected shell section. Hot work permit and confined space precautions will be required given proximity to the tank foundation.

Inspector Signature: ____________________

Date: ____________________