Suryanagar Refinery, Gujarat

Standard Operating Procedure

# P-101 Pump Startup and Shutdown Procedure

| Document Number | SOP-UNIT2-014 |
| --- | --- |
| Equipment / Area | P-101 - Centrifugal Feed Pump, Unit 2 |
| Applicable To | Unit 2 operations personnel and shift engineers responsible for feed pump operation |
| Effective Date | 2026-01-01 (Revision 3) |
| Prepared By | M. Iyer (Maintenance Head) |
| Approved By | R. Sharma (Shift Engineer) |

## 1. Purpose

To define the standard steps for safe startup and shutdown of feed pump P-101, including verification of the mechanical seal flush system, to prevent premature seal wear or failure.

## 2. Procedure Steps

- Confirm suction valve fully open and discharge valve closed before startup.

- Verify seal flush line pressure indicator reads within normal operating range (0.5-1.0 bar above pump discharge).

- Start pump motor and observe for abnormal vibration or noise during first 60 seconds.

- Slowly open discharge valve while monitoring flow and pressure gauges.

- Once flow stabilizes, log startup parameters in the shift logbook.

- For shutdown: close discharge valve gradually, then stop motor, then close suction valve.

- Confirm seal flush line remains active for 5 minutes post-shutdown to allow seal cool-down.

## 3. Precautions / Safety Requirements

- Do not run pump dry under any circumstances - verify suction conditions before start.

- Report any seal flush line pressure deviation immediately - do not defer.

- Abnormal vibration at startup requires immediate stop and inspection before restart.

## 4. References / Linked Documents

Work Order WO-2026-0098 (P-101 Seal Leak Repair); Inspection Report INS-2026-0031 (P-101 Seal Wear Follow-up)

Operator Acknowledgement: ____________________   Date: ____________________